Wayfinding

Code Overview

(written by Chat GPT after I gave it the ZIP file and my understanding of things)

Project Definitions and Overview:

Moon

- 92 cells (each hexagon or pentagon is called a cell)

- 80 active cells + 12 dummy cells

- Dummy cells are called poles (top and bottom)

- There are 8 cells in a panel

- There are 5 panels in a hemisphere

- There are 2 hemispheres (upper and lower)

- The naming convention is:

  - Cell – 0-7

  - Panel – 0,1,3,4,5 (we skip 2)

  - Hemisphere – Upper = 0, Lower = 1

- Each cell contains 3 switches, 3 LED boards, and 1 control board (STM32G030)

  - Each control board is given an address based on the above naming convention)

- Every panel has a buffer board that boosts the I2C signal

- Each hemisphere has it’s own bus

- Then the moon has a main control board with is an ESP32 Dev Board

- The moon will communicate with the moth via ESPNow

Moth

- The both has 2 servo motors

- This allows it to rotate and tilt forwards and backwards (we want to limit tilt to 45 degrees)

- The moth has LED lights embedded into the back of it.

  - There are 45 addressable LED sections

  - Each section has 14 LEDs

  - The moth is divided into 4 sections

    - Outside Right

    - Inside Right

    - Inside Left

    - Outside Left

  - But some of the 45 sections are non viewable. So we’ll need to do some testing to figure out which of the 45 belong to which section.

- There is an ESP32 S3 board inside the moth (connect to the moon’s ESP32 via ESPNow).

- This will receive data from the moon and then power the servo’s, the LEDs and will also send MIDI signal to a computer that will play music

Modes (detailed info in section 6)

- Light Mode – one press to turn on, each press cycles a colour

- Music Mode – 4 instruments can make music

- Pressure Mode – the harder you press it cycles through the colours

- Game Mode – plays the game simon

- Motion Mode – moves the moth and controls it’s LEDs

- Orbit Mode – this is a standby mode that operates when the moon isn’t touched for 2 minues

Breakdown for developers:

- Controllers (3 separate firmwares)

- Data model: cells, addressing, MIDI mapping

- Sub-controller (cell) firmware responsibilities

- Moon controller responsibilities

- Moth controller responsibilities (planned)

- Modes + behaviour summary (from the Moth Modes sheet)

- Config + calibration system

- Major “next coding steps”

## 1. Controllers (3 separate firmwares)

### A. SubController (STM32G030, one per cell)

- Project: Controllers/SubController

- Framework: PlatformIO + Arduino core for STM32

- Hardware per cell:

  - 3 analog pressure sensors: PIN_SENSE[3] = { PA0, PA2, PA1 }

  - 3 NeoPixel LEDs: PIN_LED = PA12, NUM_LEDS = 3

  - 2 status LEDs: PIN_STATUS[2] = { PC15, PC14 }

  - 7 address pins: I2C_ADDRESS_PINS[7] = { PA3, PA4, PA5, PA6, PA7, PA8, PA11 }

  - I²C: SCL = PB6, SDA = PB7

Role: “Smart cell module”

- Reads its 3 pressure sensors

- Manages its local LED behaviour (different patterns per mode)

- Exposes data + accepts commands over I²C as a slave

- Stores its own calibration/config struct in STM32 flash

### B. MoonController (ESP32 on the moon)

- Project: Controllers/MoonController

- Framework: PlatformIO + Arduino (ESP32)

- Roles:

  - I²C master talking to all SubControllers

  - Runs the web interface (WiFi AP + HTTP + WebSocket)

  - Owns a global Config struct (moon-wide and per-cell settings)

  - Sends mode / config / colour commands down to cells

  - (In future) Talks to the moth ESP32-S3 via ESP-NOW

Right now this codebase is mostly:

- Config management (MoonConfig)

- Web UI + JSON API

- Some I²C command plumbing (MainController)

The higher-level “art logic” (music, game, etc.) is just starting to be scaffolded in.

### C. MothController (ESP32-S3 on the moth)

- Project: Controllers/MothController

- Current state in this branch: PlatformIO skeleton only, no src code yet.

- Target role (based on your description, not code yet):

  - Acts as a USB-MIDI device to the computer

  - Talks to the Moon ESP32 via ESP-NOW

  - Controls:

    - 2 servos (pan 270°, tilt limited in code to ±45°)

    - 1 LED strip split into 4 logical zones (outside L / inside L / inside R / outside R)

  - Implements moth-side behaviour for the modes

  - Turns moon events (cell presses + pressures) into MIDI notes / CCs based on your spreadsheet

## 2. Data model: cells, addressing, MIDI mapping

### A. Moon cell topology (hardware)

- Physically: 2 hemispheres × 5 panels/hemisphere × 8 cells/panel

  - Total 80 wired cells (out of 92 geometric cells)

  - Hemispheres: H0 (upper), H1 (lower)

  - Panels: P0, P1, P3, P4, P5 (panel 2 unused)

  - Cells on a panel: C0–C7

Note: in MoonConfig.h it currently defines:

#define NUM_HEMISPHERES 2

#define NUM_PANELS_PER_HEMISPHERE 4

#define NUM_CELLS_PER_PANEL 8

→ That’s 2×4×8 = 64. You’ll want to align this with the 80-cell reality (2×5×8).

### B. Cell identity → MIDI mapping (from the “Moon Numbering System MIDI Channels” sheet)

For each cell, the spreadsheet gives:

- Hemi: H0 or H1

- Panel: P0, P1, P3, P4, P5

- Cell: C0–C7

- MIDI Channel (1, 2, 3, or 10)

- MIDI Note (e.g., 43, 105, …)

Example rows:

- H0 P0 C0 → Channel 2, Note 43

- H0 P1 C3 → Channel 10, Note 43

- H0 P3 C7 → Channel 3, Note 88,

- H1 P5 C3 → Channel 10, Note 40

- …etc for all 80 cells.

Takeaway for coders:

- On the moth S3, you’ll want a lookup table:

- struct CellMidiMap {

- uint8_t hemi;

- uint8_t panel;

- uint8_t cell;

- uint8_t channel;

- uint8_t note;

- bool    isModeSwitch;

- };

- When a CellEvent comes in over ESP-NOW, you look up (H,P,C) and send the corresponding MIDI note on the right channel, using the pressure as velocity or a CC.

See the MIDI mapping spreadsheet for the full MIDI mapping. Also we want the LEDs of each instrument to display a very light colour (20%?) when not pressed. So you can see where each instrument is. And then when you press it it lights up. All of the channels can be in pressure mode for the lights. Except the drums (channel 2) which will just be press and it turns the colour on once (then the colour turns off).

## 3. SubController firmware responsibilities (per cell, STM32)

Key files:

- include/pins.h – pin assignments

- include/Config.h, src/Config.c – config struct + flash read/write

- include/modes.h, src/modes.cpp – current mode (Pressure, Moth, Music, Colour, Simon Display/React, Standby, Calibration, Test, Flash)

- src/pressureSensors.cpp – reads sensors, tracks pressure and press state

- src/NeoPixel.cpp – LED patterns per mode

- src/i2cComs.cpp – I²C slave setup & handlers

- src/commands.cpp – interprets incoming commands and updates mode/config

### A. Sensor handling

- Reads all three velostat channels.

- Tracks:

  - Short- and long-term averages

  - pressureMin[3] / pressureMax[3] (for calibration)

  - A normalized pressure value (0–255)

  - Press/release state and retrigger timing

- Behaviour depends on current mode:

  - Base/Pressure vs Calibration vs Test, etc.

### B. LED behaviour

In NeoPixel.cpp:

- neopixel_init() – sets up strip, brightness, turns LEDs off.

- LED_update_mode(pressure):

  - Switches on get_mode() and calls:

    - LED_mode_base(pressure) // “Pressure mode” behaviour

    - LED_mode_moth(pressure)

    - LED_mode_music(pressure)

    - LED_mode_colour(pressure)

    - LED_mode_simon(pressure)

    - LED_mode_standby(pressure)

    - LED_mode_calibrate()

    - LED_mode_flash()

These functions implement the local visual language per mode: how a single cell’s 3 LEDs behave given its pressure value and mode.

### C. Configuration + calibration (local)

- Config struct (on STM32) holds:

  - magic, structVersion, firmwareVersion

  - pressureMin[3], pressureMax[3]

  - numLongTermAvgSamples, numShrtTermAvgSamples

  - pressThreshold, releaseThreshold, maxThreshold

  - colourHueStart, colourHueEnd

- readConfig() / writeConfig() read/write this struct to a fixed flash address.

- Various handle*Cmd functions in commands.cpp are stubs to be connected:

  - e.g. handlePressThreshCmd, handleMaxThreshCmd, handleColourHueStartCmd, etc.

So the cell knows its own calibration and thresholds and can implement consistent pressure → LED behaviour across power cycles.

### D. I²C comms

In i2cComs.cpp:

- I2C_init_STM():

  - Reads the 7 address pins and computes an I²C address for that cell.

  - Configures Wire as slave on PB6/PB7.

  - Sets:

  - Wire.onReceive(I2Creceive);

  - Wire.onRequest(I2Csend);

- I2Creceive(size):

  - Reads the command byte and any payload.

  - Passes it into commandHandler(cmd, data, size) (from commands.cpp).

- I2Csend():

  - Returns data back to the Moon (currently at least the pressure value; can be extended to a status struct).

From a coding perspective:

- Moon → SubController:

  - Commands: set mode, config values, LED colours, request types.

- SubController → Moon:

  - On request: returns pressure + status.

## 4. MoonController responsibilities (ESP32 on the moon)

Key files:

- include/MoonConfig.h, src/MoonConfig.cpp

- include/MainController.h, src/MainController.cpp

- src/MoonController.cpp

- src/wayfindingweb.html (web UI)

### A. Global config (for the whole moon)

Config struct (on ESP32), defined in MoonConfig.h:

- Global parameters:

  - longTermAvgSamples, shortTermAvgSamples

  - brightness (0–100 %)

  - retriggerTime, sampleTime

  - pressThreshold, releaseThreshold, maxThreshold

  - Moon-wide colour range: startColour, endColour

  - i2cFrequency

- Mode control:

  - modeUser // “normal” user mode

  - modeOverride // forced mode

  - modeOverrideEnabled // bool

- Plus per-cell overrides (in the parts I can’t see fully, but that’s clearly intended).

MoonConfig.cpp handles:

- loadConfig() / saveConfig() to ESP32 flash

- Deriving “effective” thresholds and pressures per cell from the base + overrides.

### B. Web server + UI

In MoonController.cpp:

- Sets up ESP32 as WiFi AP.

- Exposes:

  - HTTP routes to serve wayfindingweb.html and static chunks

  - JSON endpoints to get/set config

  - A WebSocket server for realtime updates

- The web page lets you:

  - View and edit global + per-cell config values

  - Choose modes, enable/disable override

  - Trigger test/calibration behaviours

### C. I²C master to SubControllers

In MainController.cpp:

- Contains command IDs for setting modes:

- #define CMD_SET_MODE_PRESSURE      0x10

- #define CMD_SET_MODE_MUSIC         0x11

- #define CMD_SET_MODE_SIMON_DISPLAY 0x12

- #define CMD_SET_MODE_SIMON_REACT   0x13

- #define CMD_SET_MODE_CALIBRATION   0x14

- #define CMD_SET_MODE_TEST          0x15

- #define CMD_SET_MODE_MOTH          0x16

- #define CMD_SET_MODE_COLOUR        0x17

- #define CMD_SET_MODE_STANDBY       0x18

- #define CMD_SET_MODE_FLASH         0x19

- And helper(s) like I2C_send_mode(addr, "Pressure") that:

  - Turn those high-level strings into command bytes

  - Write them over I²C using Wire.beginTransmission().

There are also placeholders for:

- I2C_send_config(...) – send per-cell config down to STM32.

- I2C_send_col(...) – send colour data.

- I2C_send_request(...) – tell the SubController what type of data to return on read.

### D. (Future) Art logic & ESP-NOW

What’s not yet implemented in this branch:

- The actual loop that:

  - Walks over all cells (H,P,C)

  - Requests their pressure/status

  - Runs the mode logic (Light/Pressure/Music/Game/Motion/Orbit)

  - Decides which I²C commands to send back.

- Any ESP-NOW communication with the moth.

Those are the big “brains” pieces to build next.

## 5. MothController responsibilities (ESP32-S3, planned)

Right now: Controllers/MothController just has a platformio.ini pointing at freenove_esp32_s3_wroom. No code yet in this branch.

Target responsibilities (from your description + sheets):

- USB-MIDI device to the computer

  - Uses your Moon MIDI map (H,P,C → channel+note).

  - Generates MIDI note on/off and CCs based on cell events and pressure.

- ESP-NOW peer with the Moon ESP32

  - Receives events:

    - (hemi, panel, cell, justPressed, justReleased, pressed, pressure, mode)

  - (Optionally) Sends commands back:

    - global mode changes, triggers, etc.

- Servo control

  - Tilt servo: limited to ±45° from centre.

  - Pan servo: up to 270°, constrained to a safe range.

  - Motion patterns differ per mode (see below).

- Moth LED strip control

  - One long strip, but logically split into 4 zones:

    - Outside left, Inside left, Inside right, Outside right.

  - Patterns per mode (breathing, reacting to moon, matching colour bands, etc.).

## 6. Modes + behaviour summary (from the “Moth Modes” sheet)

From Moth Modes.xlsx (condensed):

Columns: Mode, Moon touch, Pre-pattern, Moon LEDs, Moth LEDs, Moth Motion.

### Light Mode

- Moon touch: Cycle colour on press.

- Pre-pattern: None.

- Moon LEDs: Each press cycles the cell’s colour; final press turns it off.

- Moth LEDs: Match moon colours per quadrant (1,2,3,4 map moon zones to moth zones; wraps after 4).

- Moth motion: Stationary.

### Music Mode

- Moon touch: Pressure ramp.

- Pre-pattern: “4 × instruments 20% LED” – each instrument section lightly indicated.

- Moon LEDs: Press to play notes; pressure changes parameters (colour/brightness).

- Moth LEDs: Pulse to match music.

- Moth motion: Move subtly with the groove.

### Pressure Mode

- Moon touch: Pressure ramp.

- Pre-pattern: None.

- Moon LEDs: Colour cycles based on pressure; on release, it holds the max colour then slowly fades.

- Moth LEDs: Duplicate moon colours.

- Moth motion: Stationary.

### Game Mode (Simon)

- Moon touch: Single press.

- Pre-pattern: None.

- Moon LEDs: Flash a sequence for the game; respond to player presses (green when right, red when wrong).

- Moth LEDs: Shows excitement when you win (extra flashes, etc.).

- Moth motion: Excited movement when you win.

### Motion (Moth) Mode

- Moon touch: Cycle colour on press.

- Pre-pattern: X + Y (“+ sign”) pattern to tell people which cells move the moth.

- Moon LEDs: Indicates which cells are active for motion; likely a plus shape.

- Moth LEDs: Emphasise the “+” cells; other cells maybe dimmer.

- Moth motion: Direction/speed/magnitude depend on which parts of the “+” shape you press and how hard.

### Standby / Orbit Mode

- Moon touch: No inputs.

- Pre-pattern: None.

- Moon LEDs: Half the moon lit with white; LEDs slowly rotate around (orbit effect).

- Moth LEDs: Simple, slow light show.

- Moth motion: Gentle preprogrammed “flight” pattern, following the moon’s orbit.

## 7. Config + calibration system (both sides)

### SubController (cell, STM32)

- Local Config struct with:

  - pressureMin[3], pressureMax[3] – calibration

  - Thresholds + averaging parameters

  - Colour ranges (hue start/end)

- readConfig() on startup → load from STM32 flash.

- writeConfig() used to persist calibration/adjustments.

- Calibration mode:

  - Mode starts as MODE_CALIBRATION.

  - While in calibration, pressing the cell updates min/max.

  - LED uses colour to represent raw pressure.

  - There’s a special MODE_CALIBRATION_SAVE (aliased to TEST right now) intended to save data.

### MoonController (ESP32)

- Global Config struct for the moon:

  - Default values for thresholds, brightness, timing.

  - Mode override flags.

  - Per-cell overrides (for tuning individual cells).

- loadConfig() / saveConfig() to flash.

- Web UI:

  - Allows editing these params live.

  - Sends them out to cells via config commands over I²C.

Where Garnet comes in tomorrow:

- Wiring up the calibration flow end-to-end:

  - Start calibration mode from web UI or Moon.

  - Each SubController records min/max.

  - When calibration is done, a command tells all cells to writeConfig().

  - Moon saves its own global/per-cell config as well.

- Making sure the same calibration data is visible + editable in the web UI.

## 8. Major “next coding steps” (from a dev’s perspective)

Given all this, here are the big logical chunks still to build:

- Finish SubController command handlers

  - Implement all the handle*Cmd functions in commands.cpp to:

    - Change thresholds, averaging, hue start/end, etc.

    - Enter/exit Calibration and Test modes properly.

  - Ensure I2Csend() returns a clean CellStatus struct (pressure + flags).

- Align cell topology

  - Update NUM_PANELS_PER_HEMISPHERE (and any loops) to actually cover your full H0/H1, P0,1,3,4,5, C0–7 layout (80 cells), or explicitly document why only 64 are logically addressed.

- MoonController: implement the main mode logic

  - A loop that:

    - Polls all SubControllers for pressure/state.

    - Maintains an in-memory table of all 80 cells (status + calibration).

    - Implements the 6 modes:

      - Light Mode: edge-triggered colour cycling per cell.

      - Pressure Mode: capture max pressure per press and fade.

      - Music Mode: track pressure, send note events to moth, set per-cell LED modes.

      - Game Mode: Simon state machine, mapping logical “slices” vertically across hemispheres.

      - Motion Mode: interpret “+” touches, compute moth motion vectors.

      - Standby/Orbit Mode: global orbit pattern + global moth path.

- ESP-NOW protocol between Moon ESP32 and Moth ESP32-S3

  - Define and implement packets like:

    - CellEventPacket { hemi, panel, cell, flags, pressure, mode }

    - GlobalModePacket { mode }

  - Moon sends events; moth consumes them to drive MIDI, servos, moth LEDs.

- MothController implementation

  - USB-MIDI device (ESP32-S3, TinyUSB).

  - Load your MIDI mapping from the spreadsheet into a lookup table.

  - Use pressure as velocity/CC depending on the mode.

  - Implement servo motion patterns per mode.

  - Implement moth LED patterns per mode and per “zone”.

- Glue calibration into the web UI

  - Make sure the UI can:

    - Launch calibration.

    - Show live press feedback (red when pressed).

    - Indicate when calibration is complete & saved.

  - Expose per-cell overrides to correct outliers.